SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09_____

District:  Chico Unified School District 	Inspector: Dale Hayes

School: Chico Country Day School (FKA Fairview High School)	Building: A

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT BEIGE | 10 | 1-5% | 1,920 S | CLASSROOM 1 AND ADMINISTRATION OFFICES | (1) (4) NEEDS WAX COAT TO AVOID WEAR IN ROOM 1 | N |
| M | DRYWALL | 75 | TRACE | 1,400 S | OFFICE WALLS, RESTROOMS, AND CUSTODIAN ROOM | (1) (4) | N |
| M | 12" ACT | 101 | ASSUMED | 1,920 S | CLASSROOM 1 AND ADMINISTRATION OFFICES | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09_____

District: Chico Unified School District	Inspector: Dale Hayes

School: Chico Country Day School (FKA Fairview High School)	Building: B

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT BEIGE | 10 | 1-5% | 4,300 S | ALL CLASSROOMS & COUNSELING OFFICES | (1) (4) NEEDS WAX COAT, Room 4 needs 1 tile | N |
| M | DRYWALL | 98 | NON ASBESTOS | UNDETERMINED | CLASSROOMS AND CUSTODIAN ROOM | (13) USD sample FD 1-6 6/3/05 | N |
| M | DRYWALL | 98 | ASSUMED | 600 S | BOY’S & GIRL’S RESTROOMS CEILING | (1) (4) Boys ceiling shows minor repair | N |
| M | 12" ACT | 101 | ASSUMED | 4,220 S | ALL CLASSROOMS | (1) (4) | N |
| M | 2 X 4 FCP | 102 | ASSUMED | 50 S | CLASSROOM 2 | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09_____

District: Chico Unified School District	Inspector: Dale Hayes

School: Chico Country Day School (FKA Fairview High School) 	Building: D

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP (PLASTIC) | 96 | NOT SUSPECT | 2,400 S | ALL CLASSROOMS 6, 7, 8 & 10 | (13) CUSD sample CCDS 01 & 02 7/20/05 | N |
| M | 9" VFT | 97 | Not Detect | 2,400 S | ALL CLASSROOMS | (13) CUSD sample CCDS 01 & 02 7/20/05 | N |
| M | 2 X 4 FCP | 102 | ASSUMED | 600 S | CLASSROOM 9 ONLY | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09_____

District: Chico Unified School District	Inspector: Dale Hayes

School: Chico Country Day School (FKA Fairview High School)	Building: G - MPR

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S | PLASTER | 5 | 1-5% | 2,600 S | MPR | (1) (4) | N |
| M | 9" VFT GREEN | 6 | 1-5% | 2,800 S | MPR, KITCHEN, FOOD STORAGE, AND ENTRANCE | (2) MINOR CRACKS AND CHIPPED TILES IN THE SOUTH WEST CORNER. NEEDS WAX. | N |
| T | BOILER INSULATION | 7 | 75-80% | 150 S | BOILER ROOM | (1) (4) APPEARS TO BE IN GOOD SHAPE. | N |
| T | MUD JOINT | 8 | 35-40% | 32 EACH | BOILER ROOM | (1) (4) | N |
| M | TRANSITE PIPE FLUE | 99 | ASSUMED | 2 EACH 25 L | BOILER ROOM | (1) (4) | N |
| M | 12" VFT RED | 100 | ASSUMED | 150 S | KITCHEN | (1) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09_____

District: Chico Unified School District	Inspector: Dale Hayes

School: Chico Country Day School (FKA Fairview High School)	Building: H -  Classrooms  23-27

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S |  | CERTIFIED NON-ASBESTOS | 3,200 S | CLASSROOMS 23-25 | (13) LETTER ON FILE 2007. | N |
| M | 12" VFT WHITE/BLUE |  | CERTIFIED NON-ASBESTOS | 3,200 S | CLASSROOMS 23-25 | (13) LETTER ON FILE 2007. | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09_____

District: Chico Unified School District	Inspector: Dale Hayes

School: Chico Country Day School (FKA Fairview High School)	Building: Portable

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DRYWALL |  | Certified Non-Asbestos |  | RESTROOM | (13) Letter on file dated 10/6/06 | N |
| M | 2' X 4' FCP |  | Certified Non-Asbestos |  | RESTROOM | (13) Letter on file dated 10/6/06 | N |
| M | 12" VFT GREY |  | Certified Non-Asbestos |  | RESTROOM | (13) Letter on file dated 10/6/06 | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS