SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/30/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Corporate Yard	Building: A

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT | 2 | 1-5% | 980 S | MECHANICS OFFICE & MAINTENANCE OFFICE | (2) DAMAGED UNDER CHAIR AT DESK. NEEDS WAS | N |
| M | 12" VFT BEIGE | 4 | TRACE | 60 S | UPSTAIRS RESTROOM | (1) (4) SAMPLE HMS-COP-04 COLLECTED IN 1988. | N |
| M | 12" VFT YELLOW | 6 | TRACE | 200 S | TRANSPORTATION OFFICE, ROLLING CHAIR WEAR-OUT | (2) SAMPLE HMS-COP-06 COLLECTED IN 1988. NEEDS WAS | N |
| M | 12" ACT | 7 | NONE DETECTED | 680 S | DRIVERS LOUNGE & TRANSPORTATION OFFICE | - SAMPLE HMS-COP-07 COLLECTED IN 1988. | N/A |
| M | 12" VFT GREY | 8 | ASSUMED | 260 S | TRANSPORTATION OFFICE | (1) | N |
| S | TEXTURE | 9 | ASSUMED | 200 S | TRANSPORTATION OFFICE | (1) | N |
| M | 9" VFT PINK | 62 | ASSUMED | 500 S | KEY SMITH SHOP | (1) (4) | N |
| M | DRYWALL | 77 | NONE DETECTED | 5,000 S | ALL ROOMS | - SAMPLE HMS-CUSD-CORP-77A COLLECTED ON 5-20-92. | N/A |
| M | 12" VFT CREAM | 78 | ASSUMED | 100 S | DRIVERS LOUNGE | (1) (4) | N |
| M | GLAZING COMPOUND | 79 | ASSUMED | 50 L | GROUND SHED WINDOWS | (2) NEEDS TO BE RE-GLAZED, OLD GLAZE FALLING OUT | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/30/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Corporate Yard	Building: E - Paint Shop

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DRYWALL | 75 | 1-5% IN SKIM COAT | 2,000 S | PAINT SHOP - ALL AREAS | (1) (4) | N |

School: Corporate Yard	Building: D - Poison Storage

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DRYWALL | 76 | NONE DETECTED | 500 S | ALL ROOMS | (13) | N/A |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS