SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Loma Vista Elementary School	Building: Unit A (FKA North)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T | PIPEWRAP STRAIGHT | 4 | 70-75% | 300 L | ATTIC | (11) SAMPLE HMS-LVS-04C COLLECTED ON 3/30/88. | Y |
| M | 2' X 4' FCP | 8 | NONE DETECTED | 7,900 S | MAIN OFFICE-914, SHOP-926, BOY’S RESTROOM-917, GIRL’S RESTROOM-918, AND CLASSROOMS 5-15 | (13) SAMPLE HMS-LVS-01-6 COLLECTED ON 3/30/88. | N/A |
| M | 12" VFT OFF WHITE | 9 | 10-15% | 8,000 S | KITCHEN, GYM-11, CLASSROOMS 14 & 15, AND RESTROOM-922 | (1) (4) SAMPLE HMS-LVS-02-6 COLLECTED ON 3/30/88. | N |
| M | 12" ACT | 11 | NONE DETECTED | 2,481 S | CLASSROOM AREA 16 | (1) (13) | N/A |
| T | MUD JOINTS | 12 | <1% | 11 JOINTS | BOILER ROOM-927 | (1) (6) SAMPLE HMS-LVS-05-6 COLLECTED ON 3/30/88. | Y |
| M | 12" VFT WHITE/BLUE SPECKS | 96 | 1-5% Chrysotile | 2,481 S | CLASSROOM AREA 16 AND GYM-11 | - (4) Entek sample ECG-07-465-01A. Mastic reported non-dectected | N |
| T | BOILER LINER | 97 | 10-15% | 200 S | BOILER ROOM-927 (NO MORE BOILER) | (1) (4) (7) SAMPLE HMS-LVS-06-6 COLLECTED ON 3/30/88. | Y |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Loma Vista Elementary School	Building: Unit A (FKA North)

(Continued)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DUCT BOX | 98 | ASSUMED | 50 L | BOILER ROOM-927 (NO MORE BOILER) | (1) (4) (7) | N |
| M | TRANSITE PANELS | 99 | ASSUMED | 2,000 S | ABOVE ALL DOORS AND IN BOILER ROOM-927 | (1) (4) | N |
| M | 12" VFT CREAM | 151 | ASSUMED | 1,200 S | CUSTODIAL CLOSET-915 AND CLASSROOMS 5-9 | (1) (4) | N |
| M | 12" VFT LIGHT BLUE | 152 | ASSUMED | 50 S | CLASSROOM 9 | (1) (4) | N |
| M | 12" VFT LIGHT BLUE | 153 | ASSUMED | 175 S | CLASSROOM 5 | (7) | N/A |
| M | WHITE DUCT TAPE | 155 | ASSUMED | 100 L | BOILER UNIT EXTERIOR-927 | (1) (4) (7) | N |
| M | VIBRATION DAMPENER | 157 | ASSUMED | 20 L | BOILER ROOM-927 (NO MORE BOILER) | (1) (7) | Y |
| M | 12" VFT PINK | 158 | ASSUMED | 400 S | CLASSROOM 5 | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Loma Vista Elementary School	Buildings: Units A & B (FKA North & South)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DRYWALL | 95 | 1-5 SKIM COAT | UNDETERMINED | ALL NON-PLASTER WALLS & CEILINGS | (1) (4) | N |

School: Loma Vista Elementary School	Building: SED Portable Classroom

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DRYWALL & JOINT COMPOUND | 93 | Certified Non-Asbestos | 960 S | SED PORTABLE CLASSROOM | - DRYWALL IS ASSUMED TO EXIST BEHIND TAGBOARD. (13) Letter on file | N |
| M | 2' X 4' FCP’S FIBERGLASS | 94 | NON-SUSPECT | 900 S | SED PORTABLE CLASSROOM | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Loma Vista Elementary School	Building: Unit B (FKA South)

| Material Class | Material | Homo Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT CREAM/PINK | 1 | 1-5% | 4,100 S | HALLWAY, ORTHOPEDICS, ORTHOPEDICS CLASSROOM, PHYSICAL THERAPY-2, AND THERAPY OFFICE-900 | (1) (4) | N |
| M | 12" ACT | 2 | NONE DETECTED | 7,000 S | PSYCH INTEGRATION OFFICE, HALLWAY, PRINCIPAL’S OFFICE-908, ORTHOPEDICS, ORTHOPEDICS CLASSROOM, AND PHYSICAL THERAPY-2 | (13) | N/A |
| S | HARD PLASTER | 3 | 1-5% | 1,400 S | MEN’S & WOMEN’S RESTROOMS AND CUSTODIAN ROOM-910 | (1) (4) | N |
| S | HARD PLASTER | 5 | 1-5% | UNDETERMINED | MEN’S & WOMEN’S RESTROOMS | (1) (4) (7) REMOVED IN CUSTODIAL ROOM, STORAGE, AND NURSE’S OFFICE. | N N/A |
| M | 12" VFT WHITE WITH BLUE | 155 | ASSUMED | 200 S | OFFICE | (1) (4) BOILER ROOM WAS CONVERTED INTO A OFFICE IN JULY, 2001. | N |
| M | GYPSUM BOARD | 156 | ASSUMED | 450 S | OFFICE | (1) (4) BOILER ROOM WAS CONVERTED INTO A OFFICE IN JULY, 2001. | N |
| M | DRYWALL & JOINT COMPOUND | 159 | ASSUMED | UNDETERMINED | THROUGHOUT INTERIOR WALLS IN OFFICES AND CORRIDOR | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS