SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Marigold Elementary School	Building: Unit A - Rooms 1 & 2 (FKA 3)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT WHITE | 2 | 1-5% | 2,800 S | CLASSROOMS 1 & 2 | (1) (4) SAMPLE MAR-02 COLLECTED ON 3-30-88. | N |
| M | 2' X 4' FCP | 94 | ASSUMED | 1,800 S | CLASSROOMS 1 & 2 | (1) (4) | N |
| M | TRANSITE PANELS | 99 | ASSUMED | 300 S | ABOVE & BELOW GLASS PANELS (EXTERIOR) | (1) (4) | N |
| M | DRYWALL & JOINT COMPOUND | 105 | ASSUMED | 600 S | CLASSROOMS 1 & 2 | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Marigold Elementary School	Building: Unit B - MPR (FKA 5)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT CREAM | 2 | 1-5% | 3,160 S | KITCHEN (909), ADMIN OFFICE, MULTI PURPOSE ROOM (905), STORAGE, PIP ROOM (908), AND HEALTH OFFICE (903) | (2) (11) PRINCIPAL OFFICE AND PIP ROOM (908) COVERED WITH CARPET. ADMIN OFFICE (NORTH HALF ONLY) COVERED WITH 12" VFT BLUE. | N |
| M | COLD BOX CEILING | 6 | 30-35% | 20 S | COLD BOX | (1) (4) SAMPLE MAR-06 COLLECTED ON 3-30-88. | N |
| S | PLASTER | 75 | ASSUMED | 1,000 S | KITCHEN (909) AND RESTROOMS (907 & 907A) | (1) (4) | N |
| M | 12" ACT | 90 | ASSUMED | 2,500 S | MULTI-PURPOSE ROOM (905) | (1) (4) | N |
| M | 2' X 4' FCP | 91 | ASSUMED | 5,000 S | PRINCIPALS OFFICE, OFFICE, PIP ROOM (908), CLASSROOM A (FKA LIBRARY), AND HEALTH OFFICE (903) | (1) (4) | N |
| M | 12" VFT BLUE | 106 | NON ASBESTOS | 240 S | ADMIN OFFICE | (1) MODERNIZATION PROJECT 1996 LETTER FROM ARMSTRONG (MFG) ON FILE | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Marigold Elementary School	Building: Unit C - Rooms 3-5 (FKA 1)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT BROWN | 8 | 1-5% | 5,728 S | CLASSROOMS 3-5 | (1) (6) SAMPLE MAR-08 COLLECTED ON 3-30-88. | N |
| M | 12" ACT | 98 | ASSUMED | 5,728 S | CLASSROOMS 3-5 | (1) (4) | N |

School: Marigold Elementary School	Building: Unit D - Rooms 6A & 6B (FKA 1)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT | 95 | NON ASBESTOS | 240 S | CLASSROOMS 6A & 6B | (1) (4) MODERNIZATION 1996 MANUFACTURER LETTER ON FILE | N |
| M | 2' X 4' FCP | 96 | ASSUMED | 1,800 S | CLASSROOMS 6A & 6B | (1) (4) | N |
| M | DRYWALL & JOINT COMPOUND | 107 | ASSUMED | 2,500 S | CLASSROOMS 6A & 6B | (11) COVERED WITH TAGBOARD. | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE												SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Marigold Elementary School	Building: Unit E - Restrooms (FKA 1)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T | PIPE WRAP STRAIGHT | 10 | 60-65% | 150 L | BOILER ROOM | (1) (4) SAMPLE MAR-10 COLLECTED ON 3-30-88. | Y |
| M | 12" ACT | 97 | ASSUMED | 850 S | GIRL’S & BOY’S RESTROOMS | (2) MINOR WATER DAMAGE ON ACT’S IN BOY’S & GIRL’S RESTROOMS. | N |
| M | TRANSITE PANELS | 99 | ASSUMED | 50 S | MAIN BOILER ROOM AT BOILERS | (11) | N |
| S | PLASTER | 101 | NON ASBESTOS | 600 S | BOY’S & GIRL’S RESTROOMS AND CUSTODIAL ROOM | (2) (6) BOYS RESTROOM DAMAGED NEAR DOOR CUSD SAMPLE 7/7/08 MAR01,03-05 | N |
| T | BOILER INSULATION | 108 | ASSUMED | 2 EACH | BOILER ROOM | (1) | Y |

School: Marigold Elementary School	Building: Unit F - Rooms 7-9 (FKA 1)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT BROWN | 8 | 1-5% | 5,728 S | CLASSROOMS 7-9 | (1) MODERNIZATION 1996 | N |
| M | 12" ACT | 98 | ASSUMED | 5,728 S | CLASSROOMS 7-9 | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Marigold Elementary School	Building: Unit G - Room 10 (FKA 1)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP FIBERGLASS | 96 | Certified Non-Asbestos | 1,800 S | CLASSROOM 10 | (13) Letters on file | N |
| M | DRYWALL & JOINT COMPOUND | 109 | Certified Non-Asbestos | 1,700 S | CLASSROOM 10 | - Letters on file COVERED WITH TAGBOARD | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Marigold Elementary School	Building: Unit H (FKA 4)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT | 2 | 1-5% | 4,500 S | CLASSROOMS 11-14 AND MIDDLE RESTROOM | (1) (4) MIDDLE RESTROOM. - THE 9" VFT COULD BE UNDER THE CARPET IN CLASSROOMS 11, 13, & 14. | N |
| M | 2' X 4' FCP | 92 | NON ASBESTOS | 4,500 S | ALL ROOMS | (1) (4) CUSD SAMPLE COLLECTED 7/7/08 | N |
| M | 12" VFT | 93 | NON ASBESTOS | 900 S | ROOM 12 - LIBRARY | (11) 9" VFT WAS CONCEALED BY CARPET AND 12" VFT WAS REPLACED WITH 12" VFT WHITE WITH BLUE IN 1996 – NEW CARPET 9/09 | N |
| M | TRANSITE PANELS | 99 | ASSUMED | 300 S | ABOVE WINDOWS | (1) (4) APPROXIMATELY 18 PANELS. | N |
| M | 12" VFT WHITE W/BLUE | 150 | NON ASBESTOS | 305 S | STAFF LOUNGE (920) | (7) MODERNIZATION 1996 REMOVED AND REPLACED WITH 12" VFT GREEN. | N/A |
| M | 12" VFT GREEN | 151 | NON ASBESTOS | 305 S | STAFF LOUNGE (920) | (1) MODERNIZATION 1996 MANUFACTURER LETTER ON FILE | N |
| S | PLASTER | 152 | ASSUMED | 2,000 SF | RESTROOMS (918 & 919) AND STAFF LOUNGE (920) | (1) (4) | N |
| M | DRYWALL & JOINT COMPOUND | 153 | ASSUMED | 2,500 SF | CLASSROOMS 11-14 | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Marigold Elementary School	Building: Unit I - Rooms 15-17 (FKA Portables 18A-22)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | NOTE: ROOMS 15-17 WERE NOT INCLUDED IN THE ORIGINAL INSPECTION PREFORMED BY HMS, INC. IN 1988. (13) Letters on file |  |

School: Marigold Elementary School	Building: Unit J - Rooms 19-23 (FKA Portables 18A-22)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT | 80 | Certified Non-Asbestos | 2,400 S | PORTABLES 18B-22 | - (4) MODERNIZATION 1996 (13) Letter on file | N |
| M | DRYWALL | 81 | Certified Non-Asbestos | 10,000 S | PORTABLES 18B-22 AND RESTROOMS (923 & 924) | - COVERED WITH TAGBOARD IN CLASSROOMS 18B-22. (13) Letter on file | N |
| M | 2' X 4' FCP | 89 | Certified Non-Asbestos | 8,000 S | PORTABLES 18B-22 | - (4) (13) Letter on file | N |
| M | LINOLEUM | 100 | Certified Non-Asbestos | 500 S | RESTROOMS (923 & 924) | - (4) (13) Letter on file | N |
|  |  |  |  |  |  | NOTE: THE DISTRICT MUST OBTAIN CERTIFICATION LETTERS ON ANY PORTABLES MANUFACTURED AFTER OCTOBER 12, 1988. |  |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Marigold Elementary School	Building: Unit K - Room 18 (FKA Portables 18A-22)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT | 80 | Certified Non-Asbestos | 2,400 S | PORTABLE 18A | - (4) MODERNIZATION IN 1996 (13) Letter on file | N |
| M | DRYWALL | 81 | Certified Non-Asbestos | 10,000 S | PORTABLE 18A | (1) (4) (13) Letter on file | N |
| M | 2' X 4' FCP | 89 | Certified Non-Asbestos | 8,000 S | PORTABLE 18A | (1) (4) (13) Letter on file | N |
|  |  |  |  |  |  | NOTE: THE DISTRICT MUST OBTAIN CERTIFICATION LETTERS ON ANY PORTABLES MANUFACTURED AFTER OCTOBER 12, 1988. |  |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS

Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Marigold Elementary School	Building: Unit L - Portables 23 & 24

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT BLUE | 102 | Certified Non- Asbestos | 180 S | PORTABLES 23 & 24 | (1) (4) (13) Letter on file | N |
| M | DRYWALL & JOINT COMPOUND | 103 | Certified Non-Asbestos | 960 S | PORTABLES 23 & 24 | - COVERED WITH TAGBOARD. (13) Letter on file | N |
| M | 2' X 4' FCP’S (FIBERGLASS) | 104 | Certified Non-Asbestos | 900 S | PORTABLES 23 & 24 | (1) (4) (13) Letter on file | N |
|  |  |  |  |  |  | NOTE: THE DISTRICT MUST OBTAIN CERTIFICATION LETTERS ON ANY PORTABLES MANUFACTURED AFTER OCTOBER 12, 1988. |  |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS