SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Parkview Elementary School	Building: Unit A (FKA E) - Rooms 1-2 & 900-902

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S | CEILING PLASTER | 4 | 1-5% | 50 S | STAFF ROOM | (1) (4) | N |
| M | 12" ACT | 6 | NONE DETECTED | 2,800 S | ENTIRE WING | - REPRESENTED BY SAMPLE HMS-PKV-06 COLLECTED ON 4-1-88. | N/A |
| M | TRANSITE PANELS | 99 | ASSUMED | 70 S | BELOW WINDOWS | (1) (4) | N |
| M | LINOLEUM | 100 | ASSUMED | 80 S | CLASSROOM 1 | (12) COULD NOT LOCATE THIS MATERIAL. | N |
| M | 9" VFT GREEN/WHITE | 101 | ASSUMED | 150 S | OFFICE, HALL, AND STORAGE ROOM | - COULD NOT LOCATE THIS MATERIAL. | N |
| M | 12" VFT WHITE/BLUE | 102 | ASSUMED | 1,500 S | CLASSROOMS 1 & 2 | (1) (4) | N |
| M | 12" VFT OLIVE | 103 | NON ASBESTOS | 70 S | STORAGE (RM 1) | (1) MODERNIZATION 1993 MANUFACTURER’S LETTER ON FILE | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Parkview Elementary School	Building: Unit B - Portable 11A & 11B

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT CREAM | 12 | NON ASBESTOS | 120 S | CLASSROOMS 11A & 11B | (1) (4) MANUFACTURED AUGUST 1986 MANUFACTURER’S LETTER ON FILE | N |
| M | 2' X 4' FCP’S | 14 | ASSUMED | 900 S | CLASSROOMS 11A & 11B | (1) (4) | N |

School: Parkview Elementary School	Building: Unit C - Portables 12 & 13

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Assumed |  | PORTABLES 12 & 13 | - NEW CHECK DATE WHEN INSTALLED THESE TWO PORTABLES WERE NOT IN OUR ORIGINAL INSPECTION. MANUFACTURES DATE UNREADABLE. BOTH HAVE 12" VFT & 2' X 4' FCP’S. DISTRICT MUST CERTIFY IF MANUFACTURE DATES FOR THESE PORTABLES ARE PAST 10-12-88. |  |
|  | 12” VFT speckled off white |  | Non Asbestos | 1920 | Portables 12 & 13 | - NEW CHECK DATE WHEN INSTALLED Installed 2007 asbestos replacing original VFT |  |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Parkview Elementary School	Building: Unit D - Portable 22

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Certified Non-Asbestos | 1920 | Portable 22 | Portable 22 requires certification. Portable 22 manufactures date 1995. Letter on file, dated 9/24/97. |  |

School: Parkview Elementary School	Building: Unit E (FKA G) - Rooms 18-21

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT TWO TONE BROWN | 16 | NONE DETECTED | 640 S | CLASSROOMS 18-21 | (7) SAMPLE HMS-PKV-16 WAS COLLECTED ON 4-1-88. | N/A |
| M | 2' X 4' FCP’S | 17 | NONE DETECTED | 3,800 S | CLASSROOMS 18-21 | (13) SAMPLE HMS-PKV-17 COLLECTED ON 4-1-88. | N/A |
| T | PIPE WRAP CORNER | 18 | 5-10% | 20 S | CLASSROOM 18 | (11) ABOVE FCP’S. SAMPLE HMS-PKV-18 COLLECTED ON 4-1-88. | Y |
| M | 12" VFT WHITE/BLUE | 16 | NON ASBESTOS | 640 S | CLASSROOMS 18-21 | (1) (4) MODERNIZATION 1993 MANUFACTURER’S LETTER ON FILE | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Parkview Elementary School	Building: Unit F - Rooms 14-17 & 905-908

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S | 12 | NONE DETECTED | 4,500 S | CLASSROOMS 14-17 & LIBRARY | - SAMPLE HMS-PKV-12 COLLECTED ON 3-31-88. | N/A |
| M | 9" VFT CREAM | 13 | 1-5% | 5,000 S | CLASSROOMS 14-17 AND SER ROOM | (2) (4) CRACKED AT DOOR ENTRY | N |
| T | PIPE WRAP CORNER | 15 | 1-5% | 12 EACH | RESTROOM ATTIC | (11) ATTIC DOOR IS LOCKED. | N |
| M | DRYWALL | 76 | TRACE IN SKIM COAT | 3,000 S | ALL ROOMS | (1) (4) HMS – CUSD- PRKV- 76A-C 4/16/92 | N |
| M | 12" ACT | 97 | ASSUMED | 250 S | SER ROOM | (1) (4) | N |
| M | 12" VFT GREY | 100 | ASSUMED | 250 S | LIBRARY | (1) (4) | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Parkview Elementary School	Building: Unit G (FKA A) - Rooms 910-914

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" ACT | 85 | Certified Non-Asbestos | 700 S | OFFICE CORRIDOR | (13) Letter on file, dated 3/28/94 (7) REMOVED FROM 3 OFFICES AND SPEECH ROOM. | N N/A |
| M | 12" VFT WHITE/BLUE | 86 | Certified Non-Asbestos | 3,000 S | 3 OFFICES, SPEECH ROOM, OFFICE CORRIDOR, AND STAFF LOUNGE | (13) Letter on file (11) COVERED WITH CARPET IN SPEECH ROOM. | N |
| M | TRANSITE PANELS | 99 | ASSUMED | 1,000 S | BELOW WINDOWS | (1) (4) | N |
| M | 12" VFT BLUE | 104 | Certified Non-Asbestos | 200 S | ADMINISTRATION OFFICE | (1) (13) Letter on file, dated 3/28/94 | N |
| M | Drywall |  | Non-Detect |  | Throughout | (13) HMS – CUSD- PRKV 77A-C 4/16/92 |  |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Parkview Elementary School	Building: Unit H (FKA C) - Rooms 7-10

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DRYWALL | 82 | Certified Non-Asbestos | 800 S | MEN’S & WOMEN’S RESTROOMS | (1) (13) Letter on file, dated 3/28/94 | N |
| M | 12" VFT WHITE/BLUE | 86 | Certified Non-Asbestos | 1,000 S | CLASSROOMS 7-10 | (13) Letter on file, dated 3/28/94 | N |
| M | 2' X 4' FCP | 87 | Certified Non-Asbestos | 2,600 S | CLASSROOMS 7-10 | (13) Letter on file, dated 3/28/94 | N |
| M | TRANSITE PANELS | 99 | ASSUMED | 400 S | BELOW EXTERIOR WINDOWS | (1) (4) | N |
| M | 12" ACT | 105 | Certified Non-Asbestos | 400 S | CLASSROOMS 7-10 | (13) Letter on file, dated 3/28/94 | N |
| M | LINOLEUM | 106 | Certified Non-Asbestos | 200 S | MEN’S & WOMEN’S RESTROOMS | (13) Letter on file, dated 3/28/94 | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Parkview Elementary School	Building: Unit I (FKA D) - Rooms 914-918

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S | PLASTER | 4 | 1-5% | 1,100 S | BOY’S & GIRL’S RESTROOM WALLS AND CUSTODIAN ROOM | (1) (4) | N |
| M | DRYWALL | 83 | Certified Non-Asbestos | 300 S | BOY’S & GIRL’S RESTROOM CEILING | (13) Letter on file, dated 3/28/94 | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Parkview Elementary School	Building: Unit J (FKA B) - Rooms 3-6

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT WHITE/BLUE | 86 | Certified Non-Asbestos | 1,000 S | CLASSROOMS 3-6 AND WORKROOM | (13) Letter on file, dated 3/28/94 | N |
| M | 2' X 4' FCP | 87 | Certified Non-Asbestos | 3,200 S | CLASSROOMS 3-6 | (13) Letter on file, dated 3/28/94 REMOVED FROM WORKROOM. | N N/A |
| M | TRANSITE PANELS | 99 | ASSUMED | 400 S | BELOW WINDOWS | (1) (4) | N |
| M | 12" ACT | 107 | Certified Non-Asbestos | 400 S | CLASSROOMS 3-6 | (13) Letter on file, dated 3/28/94 | N |
| M | DRYWALL | 108 | Certified Non-Asbestos | 1,200 S | COPY ROOM AND CUSTODIAL ROOM | (13) Letter on file, dated 3/28/94 | N |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Parkview Elementary School	Building: Unit K (FKA H) - Rooms 921-925

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S | PLASTER | 3 | NONE DETECTED | 1,200 S | KITCHEN | (1) (4) (13) REPRESENTED BY SAMPLES HMS-PKV-03A-F COLLECTED ON 4-1-88. | N/A |
| M | 12" VFT RED | 9 | 1-5% | 540 S | KITCHEN (NORTH WEST CORNER) | (1) (4) | N |
| M | DRYWALL | 11 | ASSUMED | 60 S | GIRL’S & BOY’S RESTROOM CEILINGS | - (4) BOYS RESTROOM (6) GIRLS RESTROOM CEILING TAPE PEELING | N |
| M | 12" VFT WHITE/BLUE | 92 | ASSUMED | 1,800 S | MPR, KITCHEN, AND CLOSET ON EACH END OF STAGE | (1) (4) | N |
| M | 9" VFT BEIGE | 101 | ASSUMED | 200 S | KITCHEN STORAGE | (1) (2) NEEDS WAXING TO ENCAPSULATE | N |

School: Parkview Elementary School	Building: Unit L - Portables 23-25

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Certified Non-Asbestos |  | PORTABLES 23-25 | PORTABLES 23-25 REQUIRE CERTIFICATION. PORTABLES 23-25 MANUFACTURERS DATE 1997. Letter on file, dated 2/10/00 |  |

CONDITION CODE

General Condition:	(1) Good	(2)  Damaged	(3)  Significantly Damaged (25% or more)

Change in Condition:	(4) No		(5)  Yes (If Yes, explain under comments)

Abated:		(6) Repaired	(7)  Removed	(8)  Encapsulated	(9)  Enclosed   	(10) Isolated & Restricted

Miscellaneous:		(11) Inaccessible, no inspection data	    (12) Other (Explain under comments)   	 (13) Non-Asbestos/Not Inspected

CONSULT ORIGINAL AND SUPPLEMENTARY INSPECTION REPORTS FOR MATERIALS WITH TRACE AMOUNTS OF ASBESTOS