SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Rosedale Elementary School	Building: Unit A (FKA D)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT GREEN | 3 | 5-10% | 3,200 S | MPR (905), KITCHEN 908, KITCHEN STORAGE 909A, AND STORAGE ROOM 909B | (1) | N |
| M | 12" VFT | 92 | ASSUMED | 100 S | ENTRY OF OFFICE 909 | (7) MODERNIZATION 1989 | N/A |
| M | 12" VFT | 93 | NON ASBESTOS | 3,000 S | CORRIDOR | (1) (4) MODERNIZATION 1989 (7) OFFICES AND MULTI-PURPOSE ROOM MANUFACTURER’S LETTER ON FILE | N N/A |
| M | 12" VFT GREY | 107 | NON ASBESTOS | 400 S | ADMINISTRATION OFFICES 900-903, 906, 907 & 909 | (1) INSTALLED DURING THE 1989 MODERNIZATION. MANUFACTURER’S LETTER ON FILE | N |
| M | 12" ACT | 108 | ASSUMED | 400 S | ADMINISTRATION OFFICES 900-903, 906, 907 & 909 | (1) INSTALLED DURING THE MODERNIZATION. | N |
| M | LINOLEUM | 109 | ASSUMED | 100 S | RESTROOM 904 | (1) | N |

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Rosedale Elementary School	Building: Unit A (FKA D)

(Continued)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S | PLASTER | 110 | NONE DETECTED | UNDETERMINED | MPR (905) /KITCHEN 908 | (13) SAMPLE ROS01 COLLECTED ON FEBRUARY 22, 2006. | N/A |
| T | 3" O.D. PIPE RUNS | 111 | ASSUMED | 20 L | KITCHEN WATER HEATER CLOSET | (1) | Y |
| T | 3" O.D. PIPE FITTINGS | 112 | ASSUMED | 3 EACH | KITCHEN WATER HEATER CLOSET | (1) | Y |
| M | 6" COVE BASE | 113 | ASSUMED | 200 L | CORRIDOR | (1) | N |

School: Rosedale Elementary School	Building: Unit B (FKA E)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S | 94 | ASSUMED | 2,000 S | CLASSROOMS K-1, & K-2 | (1) (4) Water spots. | N |
| M | 12" VFT CREAM | 95 | NON ASBESTOS | 2,000 S | CLASSROOMS K-1, & K-2 | (1) (4) MODERNIZATION 1989 MANUFACTURERS LETTER ON FILE | N |
| M | 12" ACT | 114 | ASSUMED | 160 S | 2 CLASSROOMS K-1, & K-2 | (1) | N |
| M | LINOLEUM | 115 | ASSUMED | 120 S | 4 RESTROOMS IN EACH K-1, & K-2 | (1) | N |

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Rosedale Elementary School	Building: Unit C (FKA F) - Portable 3

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S FIBERGLASS | 103 | NON-SUSPECT | 1,200 S | PORTABLE CLASSROOM 3 | (1) (4) | N |
| M | LINOLEUM CREAM | 116 | ASSUMED | 200 S | PORTABLE CLASSROOM 3 | (1) | N |

School: Rosedale Elementary School	Building: Unit D (FKA F) - Portable 4

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S | 103 | NON SUSPECT | 1,200 S | PORTABLE CLASSROOM 4 | (1) (4) | N |
| M | LINOLEUM CREAM | 117 | NON ASBESTOS | 200 S | PORTABLE CLASSROOM 4 | (7) REPLACED IN 2008 PER CUSD SAMPLE 7/11/08 | N |
| M | LINOLEUM | 117 | NON ASBESTOS | 200S | PORTABLE CALSSROOM 4 | MANUFACTURER’S LETTER ON FILE | N |

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Rosedale Elementary School	Building: Unit E - Portables 5 & 5B

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S | 107 | ASSUMED | 600 S | CEILING OF PORTABLE CLASSROOMS 5 & 5A - RSP & READ RECOVERY | (1) (4) | N |
| M | 12" VFT CREAM | 118 | NONE DETECTED | 360 S | PORTABLE CLASSROOMS 5 & 5A | (1) REPRESENTED SAMPLE HMS-RSD-18 WAS COLLECTED ON 3-31-88 | N |

School: Rosedale Elementary School 	Building: Unit F (FKA I)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT CREAM | 18 | NONE DETECTED | 1,600 S | CLASSROOMS 6, 7, & 8 | - REPRESENTED SAMPLE HMS-RSD-18 WAS COLLECTED ON 3-31-88. | N/A |
| M | LINOLEUM | 21 | NONE DETECTED | 375 S | BOTH RESTROOMS (916 & 917) | (13) REPRESENTED SAMPLE HMS-RSD-21 WAS COLLECTED ON 3-31-88. | N/A |
| M | 2' X 4' FCP’S | 106 | ASSUMED | 1,600 S | CLASSROOMS 6, 7, & 8 | (1) (4) | N |

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Rosedale Elementary School	Building: Unit G (FKA H)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT WHITE | 18 | NONE DETECTED | 2,000 S | CLASSROOMS 9-12 | - REPRESENTED SAMPLE HMS-RSD-18 WAS COLLECTED ON 3-31-88. | N/A |
| M | LINOLEUM TAN | 21 | NONE DETECTED | 375 S | BOTH RESTROOMS (918 & 919) | (13) REPRESENTED SAMPLE HMS-RSD-21 WAS COLLECTED ON 3-31-88. | N/A |
| M | 2' X 4' FCP’S FIBERGLASS | 105 | NON-SUSPECT | 2,000 S | CLASSROOMS 9-12 | (1) (4) | N |

School: Rosedale Elementary School 	Building: Unit H (FKA G)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 9" VFT | 15 | 15-20% | 6,500 S | CLASSROOMS 13-18 | (1) (2) Room 13, 17 & 18 needs was to encapsulate tile | N |
| M | 2' X 4' FCP’S | 91 | None Detected | 6,500 S | CLASSROOMS 13-18 AND LIBRARY (920) | (13) CUSD sample PSI-RS9 dated 3/22/08 | N |
| M | DRYWALL | 101 | ASSUMED | 500 S | BOTH RESTROOMS (921 & 923) AND TV/PARENT AREA (920) | (1) | N |
| M | 12" ACT | 102 | ASSUMED | 250 S | TV/PARENT AREA (920) | (1) (4) | N |
| M | 12" VFT GREY | 119 | NON ASBESTOS | 1,200 S | LIBRARY/TV PARENT AREA (920) | (1) MODERNIZATION 1989 MANUFACTURER’S LETTER ON FILE | N |

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Rosedale Elementary School	Building: Unit I (FKA B)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DRYWALL | 97 | ASSUMED | 400 S | BOTH RESTROOMS (924 & 925) AND WORK ROOM (926) | (1) (4) RESTROOMS COVERED WITH MELOMINE | N |
| M | 12" VFT CREAM | 98 | NON ASBESTOS | 4,000 S | CLASSROOMS 19-23 AND WORK ROOM (926) | (1) (4) MANUFACTURER LETTER ON FILE MODERNIZATION PROJECT 1989 | N |
| M | 2' X 4' FCP’S | 99 | ASSUMED | 4,000 S | CLASSROOMS 19-23 | (1) (4) (7) REMOVED FROM WORK ROOM. | N N/A |
| M | 12" ACT - WALLS | 120 | ASSUMED | 200 S | CLASSROOMS 19-23 | (1) | N |
| M | 12" ACT - CEILING | 121 | ASSUMED | 900 S | CLASSROOMS 19-23 | (1) | N |
| M | COVE BASE | 122 | ASSUMED | 600 L | CLASSROOMS 19-23 | (1) | N |

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Rosedale Elementary School 	Building: Unit J (FKA A)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DRYWALL | 100 | ASSUMED | 1,900 S | BOTH RESTROOMS (928 & 930) | (1) (4) | N |
| S | PLASTER | 123 | ASSUMED | 1,200 S | CUSTODIAL ROOM AND BOILER ROOM (927 & 929) | (1) (4) | N |

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/28/09________

District: Chico Unified School District	Inspector: Dale Hayes

School: Rosedale Elementary School	Building: Unit K (FKA C)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S | PLASTER | 1 | 1-5% | 350 S | CUSTODIAN ROOM | (1) (4) | N |
| M | DRYWALL | 97 | ASSUMED | 300 S | READING OFFICE (FKA LOUNGE) (931) | (1) (4) | N |
| M | 12" VFT | 98 | NON ASBESTOS | 4,000 S | CLASSROOMS 24-28 | (1) (4) MANUFACTURERES LETTER ON FILE MODERNIZATION 1989 | N |
| M | 2' X 4' FCP’S | 99 | ASSUMED | 4,000 S | CLASSROOMS 24-28 | (1) (4) (7) REMOVED FROM RESOURCE ROOM. | N N/A |
| M | 12" ACT - WALLS | 124 | ASSUMED | 200 S | CLASSROOMS 24-28 | (1) | N |
| M | 12" ACT - CEILINGS | 125 | ASSUMED | 900 S | CLASSROOMS 24-28 | (1) | N |
| M | COVE BASE | 126 | ASSUMED | 600 S | CLASSROOMS 24-28 | (1) | N |