SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/30/09___________

District: Chico Unified School District	Inspector: Dale Hayes

School: Shasta Elementary School	Building: A - MPR

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | BUILDING A - BURNED DOWN 10-06-00, REBUILT IN 2001. CERTIFICATION LETTER ON FILE DATED 11-1-01. |  |

**Campus was renumbered in August 2006**

School: Shasta Elementary School 	Building: B - Classrooms 3, 6, & 7 (FKA 6, 9, & 10)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S | 1 | NONE DETECTED | 4,500 S | LIBRARY (922) AND CLASSROOMS 3 (FKA 6), 6 (FKA 9), AND 7 (FKA 10) | (13) REPRESENTED BY SAMPLE HMS-SHA-01 COLLECTED IN 1988. | N/A |
| M | 12" VFT BEIGE | 2 | TRACE | 2,500 S | ENTIRE WING | (1) (4) | N |
| M | DRYWALL | 80 | ASSUMED | 1,500 S | ENTIRE WING | (11) COVERED WITH TAGBOARD. | N |
| M | TRANSITE PANEL | 99 | ASSUMED | 100 S | OUTSIDE BUILDING | (1) (4) | N |

**Campus was renumbered in August 2006**

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/30/09___________

District: Chico Unified School District	Inspector: Dale Hayes

School: Shasta Elementary School	Building: C - Classrooms 4, 5, & 8-10 (FKA 7, 8, & 11-13)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2 X 4 FCP’S | 1 | NONE DETECTED | 7,000 S | OFFICE (915) AND CLASSROOMS 4 (FKA 7), 5 (FKA 8), 8 (FKA 11), 9 (FKA 12), AND 10 (FKA 13) | (13) SAMPLE HMS-SHA-01 COLLECTED IN 1988. | N/A |
| M | 12" VFT CREAM | 2 | TRACE | 600 S | OFFICE (915) AN D PRINCIPAL OFFICE | (1) (4) (11) CARPET OVER TILE IN PRINCIPAL OFFICE. | N |
| S | PLASTER | 86 | ASSUMED | 250 S | MEN’S (912) & WOMEN’S (914) RESTROOM CEILINGS | (11) PLASTER COVERED BY SHEETROCK. MOISTURE & DAMAGE NEWAR DOORWAY. | N |
| M | TRANSITE PANELS | 99 | ASSUMED | 200 S | OUTSIDE BUILDING | (1) (4) | N |
| M | DRYWALL | 112 | ASSUMED | UNDETERMINED | THROUGHOUT | (1) (11) COVERED WITH TAGBOARD IN ALL CLASSROOMS. | N |

**Campus was renumbered in August 2006**

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/30/09___________

District: Chico Unified School District	Inspector: Dale Hayes

School: Shasta Elementary 	Building: D - Portables 13, 13A, & 14 (FKA Portables 14, 14A, and 15)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S FIBERGLASS | 60 | NON-SUSPECT | 2,200 S | PORTABLES 13, 13A, AND 14 | (13) | N/A |
| M | 12" VFT WHITE | 70 | ASSUMED | 2,300 S | PORTABLES 13, 13A, AND 14 | - COULD BE UNDER CARPET. | N |

**Campus was renumbered in August 2006**

School: Shasta Elementary School	Building: E - Classroom 15 (FKA 16)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT TAN | 109 | CERTIFIED NONASBESTOS | 253 S | CLASSROOM 15 (FKA 16) | (13) MANUFACTURER’S LETTER ON FILE | N |
| M | 2' X 4' FCP’S | 110 | ASSUMED | 900 S | CLASSROOM 15 (FKA 16) | (1) (4) | N |
| M | DRYWALL | 111 | ASSUMED | 900 S | CLASSROOM 15 (FKA 16) | - DRYWALL BEHIND TAGBOARD. | N |

**Campus was renumbered in August 2006**

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/30/09___________

District: Chico Unified School District	Inspector: Dale Hayes

School: Shasta Elementary School	Building: F - Classroom 16 (FKA Portable 17)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT TAN | 109 | CERTIFIED NON ASBESTOS | 253 S | CLASSROOM 16 (FKA 17) | (13) MANUFACTURER’S LETTER ON FILE | N |
| M | 2' X 4' FCP’S | 110 | ASSUMED | 900 S | CLASSROOM 16 (FKA 17) | (1) (4) | N |
| M | DRYWALL | 111 | ASSUMED | 900 S | CLASSROOM 16 (FKA 17) | - DRYWALL BEHIND TAGBOARD. | N |

**Campus was renumbered in August 2006**

School: Shasta Elementary School	Building: G - Classroom 17 (FKA Portable 18)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 12" VFT TAN | 109 | CERTIFIED NON ASBESTOS | 253 S | CLASSROOM 17 (FKA 18) | (13) MANUFACTURER’S LETTER ON FILE | N |
| M | 2' X 4' FCP’S | 110 | ASSUMED | 900 S | CLASSROOM 17 (FKA 18) | (1) (4) | N |
| M | DRYWALL | 111 | ASSUMED | 900 S | CLASSROOM 17 (FKA 18) | - DRYWALL BEHIND TAGBOARD. | N |

**Campus was renumbered in August 2006**

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/30/09___________

District: Chico Unified School District	Inspector: Dale Hayes

School: Shasta Elementary School	Building: H - Classrooms 19-21

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S | 88 | CERTIFIED NON-ASBESTOS | 3,200 S | CLASSROOMS 19-21 | (13) THESE PORTABLES HAVE A MFG. DATE OF 4/97 (DMSI MFG.) CERTIFICATION LETTER ON FILE DATED 9-24-97. | N/A |
| M | 12" VFT WHITE/BLUE | 89 | CERTIFIED NON-ASBESTOS | 3,200 S | CLASSROOMS 19-21 | (13) THESE PORTABLES HAVE A MFG. DATE OF 4/97 (DMSI MFG.). CERTIFICATION LETTER ON FILE DATED 9-24-97. | N/A |

**Campus was renumbered in August 2006**

School: Shasta Elementary School	Building: I - Classrooms 22 & 23 (FKA PORTABLES 22-24)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S | 87 | CERTIFIED NON-ASBESTOS | 2,800 S | CLASSROOMS 22 & 23 | (13) THESE PORTABLES HAVE A 1/98 MFG. DATE. CERTIFICATION LETTER ON FILE DATED 8-19-98. | N/A |
| M | DRYWALL | 111 | CERTIFIED NON-ASBESTOS | 3,000 S | CLASSROOMS 22 & 23 | (13) CERTIFICATION LETTER ON LETTER DATED 8-19-98. (11) COULD BE BEHIND TAGBOARD. | N/A |

**Campus was renumbered in August 2006**

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/30/09___________

District: Chico Unified School District	Inspector: Dale Hayes

School: Shasta Elementary School	Building: J - Classroom 24

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S | 87 | CERTIFIED NON-ASBESTOS | 2,800 S | CLASSROOM 24 | (13) THESE PORTABLES HAVE A 1/98 MFG. DATE. CERTIFICATION LETTER ON FILE DATED 8-19-98. | N/A |
| M | DRYWALL | 111 | CERTIFIED NON-ASBESTOS | 3,000 S | CLASSROOM 24 | (13) CERTIFICATION LETTER ON FILE DATED 8-19-98. (11) COULD BE BEHIND TAGBOARD. | N/A |

**Campus was renumbered in August 2006**

School: Shasta Elementary School	Building: K - Classroom 18 (FKA Portable 4)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | 2' X 4' FCP’S | 88 | CERTIFIED NON-ASBESTOS | 3,200 S | CLASSROOM 18 (FKA 4) | (13) THESE PORTABLES HAVE A MFG. DATE OF 4/97 (DMSI MFG.). CERTIFICATION LETTER ON FILE DATED 9-24-97. | N/A |
| M | 12" VFT WHITE/BLUE | 89 | CERTIFIED NON-ASBESTOS | 3,200 S | CLASSROOM 18 (FKA 4) | (13) THESE PORTABLES HAVE A MFG. DATE OF 4/97 (DMSI MFG.). CERTIFICATION LETTER ON FILE 9-24-97. | N/A |

**Campus was renumbered in August 2006**

SIX MONTH SURVEILLANCE

HOMOGENEOUS ACBM RECORDS	Date of Inspection: __09/30/09___________

District: Chico Unified School District	Inspector: Dale Hayes

School: Shasta Elementary 									 Building: L-Classrooms 11 & 12  (From Former Fair View/Cal Campus)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | (PORTABLE 12- C5 from former Fair View | (12) Manufactures date 1989. |  |
| M | DRYWALL |  | None Detected |  | CLASSROOMS 11 & 12 | (13) CUSD samples SH 01-03 7/29/2005 | N |
| M | 2’ x 4’ FCP’s |  | Non Detected |  | CLASSROOMS 11 & 12 | (13) CUSD samples #SH01 & SH03 7/29/2005 | N |
| M | 12” VFT GREY |  | Non Detected |  | CLASSROOMS 11 & 12 | (13) CUSD sample #sh01 & sh03 7/29/2005 |  |

School:	Shasta Elementary 	Building: Unit M (Restroom)

| Material Class | Material | Homo. Mat. # | % Asb. | Ft2/L.F. | Location | Condition: Code & Comments | Friable Yes/No |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | DRYWALL |  | Certified Non-Asbestos |  | RESTROOM | (13) Letter on file dated 10/6/06 | N |
| M | 2' X 4' FCP |  | Certified Non-Asbestos |  | RESTROOM | (13) Letter on file dated 10/6/06 | N |
| M | 12" VFT GREY |  | Certified Non-Asbestos |  | RESTROOM | (13) Letter on file dated 10/6/06 | N |

**Campus was renumbered in August 2006**